Code: Dyon

Contacts:

Hyun-Tae Kim

Dmytro Yadykin

----------------------------------------------------------------------------------------------

Here is the list of the data needed from the particular device to run DYON with their counterparts in IDS that should be filled:

Coil currents:

pf_active.coil.current structure (time and data)

pf_active.coil.voltage structure (time and data)

optional: pf_active.coil.resistance (this vessel-coil mutual resistance in DYON)

Prefill gass pressure:

core_profiles.profiles_1d.neutral.pressure  and core_profiles.time vector for the data

BtR:

tf.b_field_tor_vacuum_r structure (time and data) or  tf.b_field_phi_vacuum_r structure for more recent DD (starting from 3.42)

The first device we want to test is MAST-U.

Then the goal of this year is to be able to read the data from non-eurofusion devices (like EAST, K-STAR, DIII-D). I wonder if this possibility is considered as a part of DMP project (or some other data mapping project at ITER)?

Initial case/steps

For the first attempt, we will only use the coil current data, not the coil voltage data. Also, we already have the machine description for MAST-U and non-EU devices. So, the following is the set of input data required:

pf_active.coil.current structure (time and data)

core_profiles.profiles_1d.neutral.pressure  and core_profiles.time vector for the data

tf.b_field_tor_vacuum_r structure (time and data) or  tf.b_field_phi_vacuum_r structure for more recent DD (starting from 3.42)

Gas_injection.pipe[:].flow_rate.data and gas_injection.pipe[:].flow_rate.time